- Please refer to the user manual:page 17:

- 8.Upgrade software

- 8.1 Install Driver

- Copy the CH340 driver (USB serial driver)_XP_WIN7 shared file in the CD or U disk to the computer, unzip it, and then click SETUP.EXE to install it. When the installation is successful. After connecting the PC and the instrument, you can see the USB-SERIAL CH340 under Device Manager/Port. At this point, you can upgrade the software software FlyMcu and the instrument data transmission.

- 8.2 FlyMcu Firmware download tool

- Copy the FlyMcu application from the CD or USB flash drive to your computer and double-click it to open it.

- Ⅰ.Choose serial port ：button 1( see the red circle as below)  Port，choose the COM of USB-SERIAL CH340

- Ⅱ.see the button 2 ( see the pic as below) at the bottom left , choose “DTR low reset，,RTS high level into BootLoader”(the fourth option)

- 8.3 Upgrade firmware

- Click the FlyMcu button 3. Select the HEX upgrade file of the main program and click Start Programming. The upgrading progress will be shows on the right window w. Do not perform other operations. Wait until the upgrade completed